#include <ESP8266WiFi.h>

const char* ssid = "YOUR_WIFI_NAME";

const char* password = "YOUR_WIFI_PASSWORD";

int relayPin = 13; // D7 GPIO13

WiFiServer server(80);

void setup() {

Serial.begin(115200);

pinMode(relayPin, OUTPUT);

digitalWrite(relayPin, HIGH);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("WiFi connected");

server.begin();

Serial.println(WiFi.localIP());

}

void loop() {

WiFiClient client = server.available();

if (!client) return;

while(!client.available()) delay(1);

String request = client.readStringUntil('\r');

client.flush();

if (request.indexOf("/LED=ON") != -1) {

digitalWrite(relayPin, LOW);

}

if (request.indexOf("/LED=OFF") != -1) {

digitalWrite(relayPin, HIGH);

}

client.println("HTTP/1.1 200 OK");

client.println("Content-Type: text/html");

client.println("");

client.println("<html>");

client.println("<h1>Bulb Control</h1>");

client.println("<a href=\"/LED=ON\"><button>Turn ON</button></a>");

client.println("<a href=\"/LED=OFF\"><button>Turn OFF</button></a>");

client.println("</html>");

}

| Relay Pin | Connect To |
| --- | --- |
| VCC | 5V (or Vin) |
| GND | GND |
| IN | D7 (GPIO13) |