Temperature sensor LM35 :

float temp;

int tempPin = 0;

void setup() {

Serial.begin(9600);

}

void loop() {

temp = analogRead(tempPin);

// read analog volt from sensor and save to variable temp

temp = temp * 0.48828125;

// convert the analog volt to its temperature equivalent

Serial.print("TEMPERATURE = "); Serial.print(temp); // display temperature

Serial.print("*C");

Serial.println();

delay(1000); // update sensor reading each one second

}

Connections:

| Pin | Connection |
| --- | --- |
| Left | VCC (5V) |
| Middle | Output → A0 |
| Right | GND |

Ultrasonic Sensor :

const int pingPin = 7; // Trigger Pin of Ultrasonic Sensor

const int echoPin = 6; //Echo Pin of Ultrasonic Sensor

void setup() {

Serial.begin(9600); // Starting Serial Terminal

}

void loop() {

long duration, inches, cm;

pinMode(pingPin, OUTPUT);

digitalWrite(pingPin, LOW);

delayMicroseconds(2);

digitalWrite(pingPin, HIGH);

delayMicroseconds(10);

digitalWrite(pingPin, LOW);

pinMode(echoPin, INPUT);

duration = pulseIn(echoPin, HIGH);

inches = microsecondsToInches(duration);

cm = microsecondsToCentimeters(duration);

Serial.print(inches);

Serial.print("in, ");

Serial.print(cm);

Serial.print("cm");

Serial.println();

delay(100);

}

long microsecondsToInches(long microseconds) {

return microseconds / 74 / 2;

}

long microsecondsToCentimeters(long microseconds) {

return microseconds / 29 / 2;

}

Connections:

- Connect VCC → 5V on Arduino

- Connect GND → GND

- Connect TRIG → Digital Pin 7

- Connect ECHO → Digital Pin 6

PIR sensor :

int pirsensor = 0;

void setup()

{

pinMode(2, INPUT);

pinMode(13, OUTPUT);

Serial.begin(9600);

}

void loop()

{

pirsensor = digitalRead(2);

if (pirsensor == 1) {

digitalWrite(13, HIGH);

Serial.println("Motion detected presence of object");

} else {

digitalWrite(13, LOW);

Serial.println("Motion not detected");

}

delay(1000); // Wait for 1000 millisecond(s)

}

Connections:

| PIR Pin | Connection |
| --- | --- |
| VCC | 5V |
| GND | GND |
| OUT | Pin 2 |

LED:

- Pin 13 → LED

DHT:

#include <DHT.h>

#define DHTPIN 2

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

dht.begin();

}

void loop() {

float temp = dht.readTemperature();

float hum  = dht.readHumidity();

Serial.print("Temp: ");

Serial.print(temp);

Serial.print(" C  ");

Serial.print("Humidity: ");

Serial.print(hum);

Serial.println(" %");

delay(1000);

}

| DHT11 Pin | Connect To |
| --- | --- |
| VCC | 5V (Arduino) |
| GND | GND |
| DATA | Digital Pin 2 |

Raindrop Sensor:

const int sensorMin = 0;

const int sensorMax = 1024;

void setup()

{

Serial.begin(9600);

}

void loop()

{

int sensorReading = analogRead(A0);

int range = map(sensorReading, sensorMin, sensorMax, 0, 3);

switch (range)

{

case 0:

Serial.println("RAINING");

break;

case 1:

Serial.println("RAIN WARNING");

break;

case 2:

Serial.println("NOT RAINING");

break;

}

delay(1000);

}

 VCC → 5V

 GND → GND

 AO → A0

Soil Moisture:

int sensor_pin = A0;

int output_value ;

void setup()

{

Serial.begin(9600);

Serial.println("Reading From the Sensor ...");

delay(2000);

}

void loop() {

output_value= analogRead(sensor_pin);

output_value = map(output_value,550,0,0,100);

Serial.print("Mositure : ");

Serial.print(output_value);

Serial.println("%");

delay(1000);

}

- VCC → 5V

- GND → GND

- AO → A0

LDR:

int sensorPin = A0; // select the input pin for LDR

int sensorValue = 0; // variable to store the value coming from the sensor

void setup() {

Serial.begin(9600); // sets the serial communication speed

}

void loop() {

// read the value from the sensor

sensorValue = analogRead(sensorPin);

// prints the values coming from the sensor on the screen

Serial.println(sensorValue);

delay(100);

}

🔹 📌 Connections (Voltage Divider)

| Component | Connection |
| --- | --- |
| LDR one end | 5V |
| LDR other end | A0 + resistor |
| Resistor other end | GND |

LCD:

#include <LiquidCrystal.h>

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

// sets the interfacing pins void setup()

void setup()

{

lcd.begin(16, 2); // initializes the 16x2 LCD

}

void loop()

{

lcd.setCursor(0,0);

lcd.print("16x2 LCD MODULE");

lcd.setCursor(2,1);

lcd.print("hello,world!");

}

| LCD Pin | Name | Connect To |
| --- | --- | --- |
| 1 | VSS | GND |
| 2 | VDD | 5V |
| 3 | V0 | Middle pin of potentiometer |
| 4 | RS | Pin 12 |
| 5 | RW | GND |
| 6 | E | Pin 11 |
| 11 | D4 | Pin 5 |
| 12 | D5 | Pin 4 |
| 13 | D6 | Pin 3 |
| 14 | D7 | Pin 2 |
| 15 | A (LED+) | 5V (via 220Ω resistor) |
| 16 | K (LED-) | GND |

| Pot Pin | Connect To |
| --- | --- |
| Left | GND |
| Right | 5V |
| Middle | LCD Pin 3 (V0) |

BULB ON OFF WIFI:

#include <ESP8266WiFi.h>

const char* ssid = "YOUR_WIFI_NAME";

const char* password = "YOUR_WIFI_PASSWORD";

int relayPin = 13; // D7 GPIO13

WiFiServer server(80);

void setup() {

Serial.begin(115200);

pinMode(relayPin, OUTPUT);

digitalWrite(relayPin, HIGH);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("WiFi connected");

server.begin();

Serial.println(WiFi.localIP());

}

void loop() {

WiFiClient client = server.available();

if (!client) return;

while(!client.available()) delay(1);

String request = client.readStringUntil('\r');

client.flush();

if (request.indexOf("/LED=ON") != -1) {

digitalWrite(relayPin, LOW);

}

if (request.indexOf("/LED=OFF") != -1) {

digitalWrite(relayPin, HIGH);

}

client.println("HTTP/1.1 200 OK");

client.println("Content-Type: text/html");

client.println("");

client.println("<html>");

client.println("<h1>Bulb Control</h1>");

client.println("<a href=\"/LED=ON\"><button>Turn ON</button></a>");

client.println("<a href=\"/LED=OFF\"><button>Turn OFF</button></a>");

client.println("</html>");

}

| Relay Pin | Connect To |
| --- | --- |
| VCC | 5V (or Vin) |
| GND | GND |
| IN | D7 (GPIO13) |

ThingSpeak:

#include <ESP8266WiFi.h>
#include "ThingSpeak.h"
#include "DHT.h"

#define DHTPIN D4
#define DHTTYPE DHT11

#define MQ3_PIN A0

const char* ssid = "YOUR_WIFI_NAME";
const char* password = "YOUR_WIFI_PASSWORD";

unsigned long channelID = YOUR_CHANNEL_ID;
const char* writeAPIKey = "YOUR_WRITE_API_KEY";

WiFiClient client;
DHT dht(DHTPIN, DHTTYPE);

void setup() {
Serial.begin(115200);
dht.begin();

WiFi.begin(ssid, password);
Serial.print("Connecting");

while (WiFi.status() != WL_CONNECTED) {
delay(500);
Serial.print(".");
}

Serial.println("\nConnected to WiFi");

ThingSpeak.begin(client);
}

void loop() {

float humidity = dht.readHumidity();
float temperature = dht.readTemperature();

int smokeValue = analogRead(MQ3_PIN);

Serial.print("Temperature: ");
Serial.println(temperature);

Serial.print("Humidity: ");
Serial.println(humidity);

Serial.print("Smoke Level: ");
Serial.println(smokeValue);

ThingSpeak.setField(1, temperature);
ThingSpeak.setField(2, humidity);
ThingSpeak.setField(3, smokeValue);

ThingSpeak.writeFields(channelID, writeAPIKey);

delay(15000); // ThingSpeak limit
}

| DHT11 Pin | Connect To |
| --- | --- |
| VCC | 3.3V (ESP8266) |
| GND | GND |
| DATA | D4 (GPIO2) |