1.LED

#define LED_PIN 8

#define BUTTON_PIN 7

void setup() {

pinMode(LED_PIN, OUTPUT);

pinMode(BUTTON_PIN, INPUT);

}

void loop() {

if (digitalRead(BUTTON_PIN) == HIGH) {

digitalWrite(LED_PIN, HIGH);

}

else {

digitalWrite(LED_PIN, LOW);

}

}

Toggling

#define LED_PIN 8

#define PUSH_BUTTON 7

int ledState = LOW;

int lastButtonState = LOW;

void setup() {

pinMode(LED_PIN, OUTPUT);

pinMode(PUSH_BUTTON, INPUT);

}

void loop(){

int currentButtonState = digitalRead(PUSH_BUTTON);

if (currentButtonState == HIGH && lastButtonState == LOW) {

ledState = !ledState;

digitalWrite(LED_PIN, ledState);

delay(50);

}

lastButtonState = currentButtonState;

}

2.Temp sensor
float temp;

int tempPin = 0;

void setup(){

Serial.begin(9600);

}

void loop(){

temp = analogRead(tempPin);

Serial.print("TEMPREATURE =");

Serial.print(temp);

Serial.print("*C");

Serial.println();

delay(1000);

}

3.Ultrasonic sensor

const int pingPin = 7;

const int echoPin = 6;

void setup(){

Serial.begin(9600);

pinMode(pingPin, OUTPUT);

pinMode(echoPin, INPUT);

}

void loop (){

long duration , inches, cm;

digitalWrite(pingPin, LOW);

delayMicroseconds(2);

digitalWrite(pingPin, HIGH);

delayMicroseconds(10);

digitalWrite(pingPin, LOW);

duration = pulseIn(echoPin, HIGH);

inches = microsecondsToInches(duration);

cm = microsecondsToCentimeters(duration);

Serial.print(inches);

Serial.print("in, ");

Serial.print(cm);

Serial.print("cm");

Serial.println();

delay(100);

}

long microsecondsToInches(long microseconds) {

return microseconds / 74 / 2;

}

long microsecondsToCentimeters(long microseconds) {

return microseconds / 29 / 2;

}

4. PIR sensor

int pirsensor = 0;

void setup(){

pinMode(2, INPUT);

pinMode(13, OUTPUT);

Serial.begin(9600);

}

void loop(){

pirsensor = digitalRead(2);

if (pirsensor == 1){

digitalWrite(13, HIGH);

Serial.println("motion detected, persence of object");

}

else {

digitalWrite(13, LOW);

Serial.println("motion not detected");

}

delay(1000);

}

6.DHT11 ESP 8266

#include <ESP8266WiFi.h>

#include "ThingSpeak.h"

#include "DHT.h"

#define DHTPIN D4

#define DHTTYPE DHT11

#define MQ3_PIN A0

const char* ssid = "Vipul";

const char* password = "00000000";

unsigned long channelID = 3327380;

const char* writeAPIKey = "9X7HMZENZX10OLOQ";

WiFiClient client;

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(115200);

dht.begin();

WiFi.begin(ssid, password);

Serial.print("Connecting");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nConnected to WiFi");

ThingSpeak.begin(client);

}

void loop() {

float humidity = dht.readHumidity();

float temperature = dht.readTemperature();

int smokeValue = analogRead(MQ3_PIN);

Serial.print("Temperature: ");

Serial.println(temperature);

Serial.print("Humidity: ");

Serial.println(humidity);

Serial.print("Smoke Level: ");

Serial.println(smokeValue);

ThingSpeak.setField(1, temperature);

ThingSpeak.setField(2, humidity);

ThingSpeak.setField(3, smokeValue);

ThingSpeak.writeFields(channelID, writeAPIKey);

delay(15000); // ThingSpeak limit

}

6. Rain drop sensor

const int sensorMin = 0; const int sensorMax = 1024; void setup()

{

Serial.begin(9600);

}

void loop()

{

int sensorReading = analogRead(A0);

int range = map(sensorReading, sensorMin, sensorMax, 0, 3); switch (range)

{

case 0: Serial.println("RAINING"); break;

case 1:

Serial.println("RAIN WARNING"); break;

case 2:

Serial.println("NOT RAINING"); break;

}

delay(1000);

Soil Moisture Sensor:

Analog Mode

int sensor_pin = A0; int output_value ; void setup() { Serial.begin(9600);

Serial.println("Reading From the Sensor ..."); delay(2000);

}

void loop() {

output_value= analogRead(sensor_pin); output_value =

map(output_value,550,0,0,100); Serial.print("Mositure : ");

Serial.print(output_value);

Serial.println("%"); delay(1000);

}

Digital Mode

int led_pin =13;

int sensor_pin =8; void setup() {

pinMode(led_pin, OUTPUT); pinMode(sensor_pin, INPUT);

}

void loop()

{

if(digitalRead(sensor_pin) == HIGH)

{

digitalWrite(led_pin, HIGH); } else

{

digitalWrite(led_pin, LOW); delay(1000);

}}}

interface LDR Sensor

int sensorPin = A0;

// select the input pin for LDR int sensorValue = 0;

// variable to store the value coming from the sensor void setup()

{

Serial.begin(9600); //sets serial port for communication

}

void loop()

{

sensorValue = analogRead(sensorPin);

// read the value from the sensor Serial.println(sensorValue);

//prints the values coming from the sensor on the screen delay(100);

}

Motor Spin

int enablePin = 9;

int in1 = 8;

int in2 = 7;

int potPin = A0;

int buttonPin = 2;

int motorSpeed = 0;

bool direction = true;

bool lastButtonState = HIGH;

void setup() {

pinMode(enablePin, OUTPUT);

pinMode(in1, OUTPUT);

pinMode(in2, OUTPUT);

pinMode(buttonPin, INPUT_PULLUP);

}

void loop() {

// Read potentiometer

int potValue = analogRead(potPin);

motorSpeed = map(potValue, 0, 1023, 0, 255);

// Button press detection

bool buttonState = digitalRead(buttonPin);

if (buttonState == LOW && lastButtonState == HIGH) {

direction = !direction;

delay(200); // debounce

}

lastButtonState = buttonState;

// Direction control

if (direction) {

digitalWrite(in1, HIGH);

digitalWrite(in2, LOW);

} else {

digitalWrite(in1, LOW);

digitalWrite(in2, HIGH);

}

// Speed control

analogWrite(enablePin, motorSpeed);

}

LCD Interface

#include <LiquidCrystal.h>

// sets the interfacing pins (RS, E, D4, D5, D6, D7)

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

void setup() {

lcd.begin(16, 2); // initializes the 16x2 LCD

}

void loop() {

lcd.setCursor(0, 0);

lcd.print("16x2 LCD MODULE");

lcd.setCursor(2, 1);

lcd.print("hello, world!");

}

Relay control

#include <ESP8266WiFi.h>

const char* ssid = "Falcon";

const char* password = "falcon@2025";

int relayPin = 13; // D7 GPIO13

WiFiServer server(80);

void setup() {

Serial.begin(115200);

pinMode(relayPin, OUTPUT);

digitalWrite(relayPin, HIGH);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("WiFi connected");

server.begin();

Serial.println(WiFi.localIP());

}

void loop() {

WiFiClient client = server.available();

if (!client) return;

while(!client.available()) delay(1);

String request = client.readStringUntil('\r');

client.flush();

if (request.indexOf("/LED=ON") != -1) {

digitalWrite(relayPin, LOW);

}

if (request.indexOf("/LED=OFF") != -1) {

digitalWrite(relayPin, HIGH);

}

client.println("HTTP/1.1 200 OK");

client.println("Content-Type: text/html");

client.println("");

client.println("<html>");

client.println("<h1>Bulb Control</h1>");

client.println("<a href=\"/LED=ON\"><button>Turn ON</button></a>");

client.println("<a href=\"/LED=OFF\"><button>Turn OFF</button></a>");

client.println("</html>");

delay(1); // Small delay to give the web browser time to receive the data

} // <--- Add this missing closing bracket!

Home automation

#include <Servo.h>

Servo door;

int pirPin = 2;

int trigPin = 10;

int echoPin = 11;

int motorEnable = 9;

int in1 = 8;

int in2 = 7;

int tempPin = A0;

int ledMotion = 3;

int ledTemp = 4;

void setup() {

pinMode(pirPin, INPUT);

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(in1, OUTPUT);

pinMode(in2, OUTPUT);

pinMode(motorEnable, OUTPUT);

pinMode(ledMotion, OUTPUT);

pinMode(ledTemp, OUTPUT);

door.attach(6);

Serial.begin(9600);

}

void loop() {

//PIR Sensor

int motion = digitalRead(pirPin);

digitalWrite(ledMotion, motion);

//  Ultrasonic

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

long duration = pulseIn(echoPin, HIGH);

int distance = duration * 0.034 / 2;

if (distance < 20) {

door.write(90); // open

} else {

door.write(0); // close

}

//  Temperature (LM35)

int tempValue = analogRead(tempPin);

float voltage = tempValue * (5.0 / 1023.0);

float temperature = voltage * 100;

if (temperature > 30) {

digitalWrite(in1, HIGH);

digitalWrite(in2, LOW);

analogWrite(motorEnable, 200);

digitalWrite(ledTemp, HIGH);

} else {

digitalWrite(in1, LOW);

digitalWrite(in2, LOW);

digitalWrite(ledTemp, LOW);

}

Serial.print("Temp: ");

Serial.println(temperature);

delay(500);

}